CAN Bus App Test Procedure

Contents

1. Introduction	3

2. Pre-Test Setup	3

3. Profiles (Required Setup)	4

4. Build and Deploy	5

5. Robot-Only Functional Tests (No PC tool)	6

5.1 Profiles and Device Activation	6

5.2 Reporting and Health	6

6. PC Tool Tests (With CAN Sniffer)	11

7. JSON + AI Report Test	12

8. Negative / Error Conditions (Optional)	13

9. Pass / Fail Criteria	13

# 1. Introduction

This document is the full test procedure for the existing code base. Use it to verify robot-only behavior and robot+PC sniffer behavior before any migration work begins.

# 2. Pre-Test Setup

1. Ensure roboRIO image and firmware are current.

2. Confirm vendor libraries installed (REV + CTRE) in the project.

3. Confirm `src/main/deploy/bringup_profiles.json` matches the hardware in front of you.

4. Ensure the Xbox controller is connected and detected.

5. Power the robot with CAN wiring complete and two terminations only.

# 3. Profiles (Required Setup)

Profiles define the hardware configuration the bringup code will control and monitor. Each profile lists the CAN devices, IDs, and labels for a specific robot setup.

Key concepts:

- Multiple profiles can be defined in the file (practice bot, comp bot, bench setup).

- The app can scroll through profiles to select one.

- The selected profile defines the known device list (what devices exist for this test).

- Selecting a profile does not activate any devices.

- Devices become active only when you explicitly activate them (one at a time or all at once).

- Active devices are the ones actually driven/commanded and included in bringup actions.

Purpose:

- Keep hardware configuration data-driven (no code edits).

- Support multiple configurations (practice bot, comp bot, bench setup).

- Ensure reports and diagnostics match the actual hardware on the bus.

Files:

- `src/main/deploy/bringup_profiles.json` (active profiles)

- `src/main/deploy/bringup_profiles.template.json` (starting template)

How to add or change a profile:

1. Open `src/main/deploy/bringup_profiles.json` (or copy the template file to this path).

2. Add a new profile entry with a unique name.

3. Fill in device lists and CAN IDs for your hardware.

4. Set `default_profile` to the profile you want at startup.

5. Deploy to the roboRIO.

File handling notes:

- This file is deployed with the robot code. Any edits require redeploy.

- Keep labels short and unique (used in reports).

- If you remove a device from the profile but it is still on the bus, it will appear as unknown in PC sniffer views (if enabled).

# 4. Build and Deploy

1. Build: `./gradlew build` (or deploy via WPILib VS Code).

2. Deploy to roboRIO.

3. Enable robot in teleop.

# 5. Robot-Only Functional Tests (No PC tool)

## 5.1 Profiles and Device Activation

1. Profile toggle (cycle)

- Action: Press `Back`.

- Expected: Profile name changes; the known device list updates.

2. Profile activate (explicit)

- Action: Use the profile-activate command for the selected profile (if present in the current build).

- Expected: The selected profile becomes the active profile (activation commands now apply to this profile).

3. Add motor (single)

- Action: Press `A`.

- Expected: Activates one device from the selected profile; should move when joystick input is applied.

4. Add all devices

- Action: Press `Start`.

- Expected: Activates all devices in the selected profile; verify with joystick input.

## 5.2 Reporting and Health

1. Print bindings

- Action: Press `Left Bumper`.

- Expected: Prints current bindings list.

- Example output:

Bindings (Operational):

A: add motor (alternates SPARK/CTRE)

Start: add all motors + CANCoders

Back: toggle CAN profile

Left Y: NEO/FLEX speed

Right Y: KRAKEN/FALCON speed

Left Stick: nudge motors (0.2 for 0.5s)

Bindings (Status):

B: print state

Left Bumper: reprint bindings

Right Bumper: print CANCoder absolute positions

D-pad Left: print health status

D-pad Down: print NetworkTables diagnostics (RobotV2 only)

D-pad Up: print CAN diagnostics report

D-pad Right: print speed inputs

X: dump CAN report JSON

2. Print health status

- Action: Press `D-pad Left`.

- Expected: For each configured device, prints local health (present, faults, warnings, voltage, motorCurrent, temp, cmdDuty/appliedDuty).

- Verify: present=YES for configured devices, busV ~12V, motorCurrentA near 0 when idle.

- Example output:

=== Bringup Health (Local Robot Data) ===

NEO index 0 CAN 10 faults=0x0 warnings=0x0 busV=12.32V cmdDuty=0.71 appliedDuty=0.71 motorCurrentA=0.92 tempC=26.0 follower=N

NEO index 1 CAN 25 faults=0x0 warnings=0x0 busV=12.31V cmdDuty=0.71 appliedDuty=0.71 motorCurrentA=1.79 tempC=26.0 follower=N

FLEX index 0 CAN 11 faults=0x0 warnings=0x0 busV=12.27V cmdDuty=0.71 appliedDuty=0.71 motorCurrentA=0.00 tempC=28.0 follower=N

roboRIO CAN 0: present=YES (virtual, no API)

======================

3. Print inputs

- Action: Press `D-pad Right`.

- Expected: Shows joystick values / requested duty.

- Example output:

Inputs: leftY=0.71 rightY=0.00 deadband=0.05

cmdDuty: spark=0.71 ctre=0.00

4. Print CAN diagnostics report (robot data only)

- Action: Press `D-pad Up`.

- Expected: CAN bus health section prints utilization, RX/TX errors, TX full, bus-off, sample age.

- Verify: counts stable (no rapid increase).

- Example output:

=== CAN Diagnostics Report ===

=== CAN Bus Diagnostics ===

Utilization: 10.5%

RX errors: 0 (delta 0)

TX errors: 0 (delta 0)

TX full: 0 (delta 0)

Bus off count: 0 (delta 0)

Sample age: 0.02s

===========================

Bus Health: (see CAN Bus Diagnostics summary above)

Device Health (local API):

NEO CAN 10: present=YES faults=0x0 sticky=0x0 warnings=0x0 stickyWarn=0x40 lastErr=kOk reset=YES busV=12.61 motorCurrentA=0.00 tempC=26.0

FLEX CAN 11: present=YES faults=0x0 sticky=0x0 warnings=0x0 stickyWarn=0x40 lastErr=kOk reset=YES busV=12.49 motorCurrentA=0.00 tempC=27.0

==============================

5. Dump JSON report (robot)

- Action: Press `X`.

- Expected: JSON printed to console and written to `/home/lvuser/bringup_report.json`.

- Verify: file exists on roboRIO after test.

- Example output:

{"timestamp":1.772000000000E9,"bus":{"valid":true,"utilizationPct":10.5,"rxErrors":0,"txErrors":0,"busOff":0},"pc":{"openOk":false,"heartbeatAgeSec":-1.0},"devices":[{"type":"NEO","id":10,"present":true,"faultsRaw":0,"warningsRaw":0,"busV":12.61,"motorCurrentA":0.0,"tempC":26.0}]}

6. Nudge motors

- Action: Press `Left Stick`.

- Expected: Motors briefly nudge at low duty.

- Verify: applied duty > 0 and motorCurrentA > 0.

7. CANCoder read

- Action: Press `Right Bumper`.

- Expected: Prints absolute position (rotations/degrees).

- Example output:

CANCoder CAN 12: absRot=0.521 absDeg=187.6 lastErr=OK

8. Print state

- Action: Press `B`.

- Expected: Prints the current robot bringup state summary.

- Example output:

State: profile=club_022426 activeDevices=3 mode=TELEOP enabled=YES

Motors: spark=2 ctre=1 cancoder=1 candle=0

9. Toggle dashboard updates

- Action: Press `Y`.

- Expected: Toggles dashboard/shuffleboard updates on/off (no change to motor behavior).

- Example output:

Dashboard updates: OFF

10. Teleop disable behavior

- Action: Disable robot in Driver Station, then re-enable.

- Expected: Motors stop while disabled. Profile selection remains but outputs are inactive until re-enabled.

# 6. PC Tool Tests (With CAN Sniffer)

Preconditions: CANable connected, slcan configured, PC tool installed.

1. Start PC bridge

- Command: `python tools/can_nt_bridge.py --rio <robot_ip> --profile <profile> --publish-can-summary --publish-unknown`

- Expected: Bridge connects and prints a summary.

2. Print PC diagnostics

- Action: Press `D-pad Down`.

- Expected: Table with ageSec, msgCount, status, etc.

- Verify: openOk=YES, heartbeat age small, fps > 0, devices show OK with recent lastSeen.

- Example output:

PC Tool: openOk=YES heartbeatAge=0.12s fps=395.8 total=68124 readErr=0 lastFrameAge=0.01s

dev 5/2/10 label=NEO 10 status=OK ageSec=0.05 msgCount=18234

dev 4/2/11 label=FLEX 11 status=OK ageSec=0.06 msgCount=18102

dev 1/7/12 label=CANCoder 12 status=OK ageSec=0.10 msgCount=9123

3. Print CAN diagnostics report

- Action: Press `D-pad Up`.

- Expected: Includes PC tool status in report, with frames/sec, total, etc.

- Example output:

PC Tool:

Status: OK

Heartbeat age: 0.12s

Open OK: YES

Frames/sec: 395.8

Frames total: 68124

Read errors: 0

Last frame age: 0.01s

Missing devices (PC): 0

Flapping devices (PC): 0

4. Unknown device detection

- Expected: Unknown device shows label=UNKNOWN in D-pad Down table when publish-unknown is enabled.

# 7. JSON + AI Report Test

1. Press `X` (dump JSON).

2. Copy `/home/lvuser/bringup_report.json` to PC.

3. Confirm file is valid JSON and includes:

- bus section

- pc section (if tool running)

- devices list

- Example output:

{

"timestamp": 1.772000000000E9,

"bus": { "valid": true, "utilizationPct": 10.5, "rxErrors": 0, "txErrors": 0, "busOff": 0 },

"pc": { "openOk": false, "heartbeatAgeSec": -1.0 },

"devices": [ { "type": "NEO", "id": 10, "present": true, "faultsRaw": 0 } ]

}

# 8. Negative / Error Conditions (Optional)

1. Remove a device from the profile but leave it physically on CAN.

- Expected: D-pad Down shows it as unknown if publish-unknown is enabled.

2. Disconnect PC tool.

- Expected: pc.openOk=false, heartbeat stale, PC fields show NaN or missing.

3. Duplicate CAN ID (if safe to test).

- Expected: devices may flap or show missing; bus errors might rise.

# 9. Pass / Fail Criteria

- All bindings trigger the correct output.

- Health report matches actual hardware state.

- JSON report is generated and valid.

- PC tool diagnostics appear only when tool is running.

- No unexpected crashes or exceptions.